Smart Glasses Firmware

Compile & Flash Guide

ESP-IDF v5.5.1  |  ESP32-U4WDH (PICO-D4)

February 2026

# 1. Hardware Reference

Your smart glasses use the following hardware:

| Parameter | Value |
| --- | --- |
| MCU | ESP32-U4WDH (revision v3.1) |
| Package | ESP32-PICO-D4 variant, QFN 5x5mm |
| Flash | 4MB embedded (XMC, device ID 4016h) |
| Crystal | 40 MHz (integrated) |
| CPU | Dual-core Xtensa LX6, up to 240 MHz |
| MAC Address | 28:05:A5:B2:1B:D4 |
| BLE Device Name | Smart_Glasses |

# 2. Prerequisites

## 2.1 Install ESP-IDF v5.5.1

Step 1: Download the ESP-IDF Tools Installer from:

https://docs.espressif.com/projects/esp-idf/en/v5.5.1/esp32/get-started/

Step 2: Run the installer and select ESP-IDF v5.5.1

Step 3: Default install location: C:\Espressif

Step 4: After installation, you will have ESP-IDF 5.5 CMD in your Start Menu

NOTE All commands in this guide must be run from the ESP-IDF 5.5 CMD prompt (not regular Command Prompt or PowerShell).

## 2.2 Project Location

The project folder should be located at:

C:\Users\dgrec\Downloads\Code-Glasses

The project folder structure:

Code-Glasses/ CMakeLists.txt (project config - names output ESP32_Ble) sdkconfig (build settings) sdkconfig.defaults (default settings: BLE enabled) idf_component.yml (dependencies: led_strip) Kconfig.projbuild (menu config options) main/ CMakeLists.txt (source file list) main.c (YOUR FIRMWARE CODE) led.c (LED control) led.h (LED header) heart_rate.h (heart rate header) heart_rate_mock.c (heart rate mock)

## 2.3 Flash Download Tool

Download flash_download_tool_3.9.9_R2 from Espressif:

https://www.espressif.com/en/support/download/other-tools

# 3. Compile New Firmware

## 3.1 Replace main.c

Step 1: Copy your new main.c file to the project's main folder:

C:\Users\dgrec\Downloads\Code-Glasses\main\main.c

This overwrites the existing main.c. No other files need to change for application logic updates.

## 3.2 Build the Firmware

Step 1: Open ESP-IDF 5.5 CMD from Start Menu

Step 2: Navigate to the project directory:

cd C:\Users\dgrec\Downloads\Code-Glasses

Step 3: Set the target chip (first time only):

idf.py set-target esp32

WARNING Running set-target will erase your build directory. Only run this the first time or if switching chip targets. Skip this step on subsequent builds.

Step 4: Build the firmware:

idf.py build

Build takes 2-5 minutes first time, under 30 seconds for subsequent builds (only recompiles changed files).

Step 5: Verify build success. You should see:

Project build complete. To flash, run: idf.py flash or run idf.py -p PORT flash

## 3.3 Locate Output Files

After a successful build, the binary files are located at:

| File Path | Flash Address | When to Flash |
| --- | --- | --- |
| build\bootloader\bootloader.bin | 0x1000 | First time only |
| build\partition_table\partition-table.bin | 0x8000 | First time only |
| build\ESP32_Ble.bin | 0x10000 | Every update |

TIP For routine firmware updates after the initial flash, you only need to flash ESP32_Ble.bin at 0x10000. The bootloader and partition table rarely change.

# 4. Flash Firmware to Board

## 4.1 Connect Programmer to PCB

Connect the programmer to the download points on the glasses PCB. The programmer must have at minimum these connections:

| Programmer Pin | ESP32 Pin | PCB Pad | Purpose |
| --- | --- | --- | --- |
| TX | U0_RXD (pin 40) | PAD127_40 | Data to ESP32 |
| RX | U0_TXD | PAD9_2 (bottom) | Data from ESP32 |
| GND | GND | PAD135_49 | Common ground |
| BOOT (IO0) | GPIO0 (pin 23) | PAD123_23 | Boot mode select |
| EN (RST) | CHIP_PU (pin 9) | PAD119_9 | Reset / Enable |

WARNING Power: Use EITHER battery OR programmer 3.3V, NOT both at the same time. If battery is connected, do NOT connect the programmer's 3.3V line. Always connect GND regardless of power source.

## 4.2 Enter Download Mode

If your programmer has auto-reset (DTR/RTS), this happens automatically. Otherwise manually:

Step 1: Hold GPIO0 (BOOT) low (connect to GND)

Step 2: Briefly pull CHIP_PU (EN) low, then release (reset pulse)

Step 3: Release GPIO0 (BOOT)

The chip is now in download mode, ready to receive firmware.

## 4.3 Flash Using Download Tool

Step 1: Open flash_download_tool_3.9.9_R2.exe

Step 2: Select Chip Type: ESP32, click OK

Step 3: Configure the bin files (check the box for each):

| File | Address |
| --- | --- |
| build\bootloader\bootloader.bin | 0x1000 |
| build\partition_table\partition-table.bin | 0x8000 |
| build\ESP32_Ble.bin | 0x10000 |

Step 4: Configure SPI Flash settings:

| Setting | Value |
| --- | --- |
| SPI SPEED | 40MHz |
| SPI MODE | DIO |
| DoNotChgBin | Checked (preserves bin headers as-is) |
| COM | Your programmer's COM port (e.g. COM18) |
| BAUD | 115200 |

Step 5: Click ERASE first (recommended before first flash or when switching firmware)

Step 6: Click START and wait for green progress bar to complete

Step 7: Wait for FINISH message

## 4.4 Verify Flash (Optional)

Step 1: Go to chipInfoDump tab in the Flash Tool

Step 2: Set Address: 0x1000, Size: 0x100

Step 3: Click Read Flash - first byte should be 0xE9 (bootloader magic byte)

## 4.5 Boot the Firmware

Step 1: Close the Flash Download Tool

Step 2: 
      Disconnect BOOT (GPIO0) from GND - must float high for normal boot

Step 3: Power cycle the board (disconnect and reconnect battery)

Step 4: The ESP32 should now boot and start BLE advertising as Smart_Glasses

# 5. Quick Firmware Update Workflow

Once initial setup is complete, updating firmware is simple:

REM 1. Replace main.c in project folder copy /Y new_main.c C:\Users\dgrec\Downloads\Code-Glasses\main\main.c REM 2. Open ESP-IDF 5.5 CMD and build cd C:\Users\dgrec\Downloads\Code-Glasses idf.py build REM 3. Flash ONLY the app binary (ESP32_Ble.bin at 0x10000) REM - Uncheck bootloader.bin and partition-table.bin in Flash Tool REM - Only check ESP32_Ble.bin at 0x10000 REM - Click START

TIP You do NOT need to erase or reflash the bootloader and partition table for routine code updates. Only ESP32_Ble.bin needs to be updated.

# 6. Serial Monitor (Debugging)

To see boot log output and debug messages from the firmware:

## 6.1 Using Arduino IDE

Step 1: Open Arduino IDE

Step 2: Tools > Board > esp32 > ESP32 PICO-D4

Step 3: Tools > Port > COM18 (your programmer's COM port)

Step 4: Tools > Serial Monitor (Ctrl+Shift+M)

Step 5: Set baud rate to 115200 (dropdown at bottom right)

Step 6: 
      Disconnect BOOT pin from GND

Step 7: Power cycle the board and watch for output

## 6.2 Using ESP-IDF Monitor

Run from the project directory:

cd C:\Users\dgrec\Downloads\Code-Glasses idf.py -p COM18 monitor

Press Ctrl+] to exit the monitor.

NOTE Close any other programs using the COM port before opening the serial monitor. Only one application can use the COM port at a time. The Flash Download Tool must be closed first.

# 7. Troubleshooting

## 7.1 Build Errors

| Problem | Solution |
| --- | --- |
| "No such file or directory" for includes | Run: idf.py set-target esp32 Then: idf.py build |
| CMake errors | Delete build folder and rebuild: rd /s /q build idf.py build |
| "command not found: idf.py" | Use ESP-IDF 5.5 CMD prompt, not regular CMD |

## 7.2 Flash Errors

| Problem | Solution |
| --- | --- |
| "Connecting..." loops forever | Ensure GPIO0 is held LOW during reset. Check TX/RX wiring (must be crossed). Try lower baud rate. |
| Flash completes but board is dead | Disconnect BOOT (GPIO0) from GND. Power cycle the board. Check battery voltage (should be 3.3-4.2V). |
| Wrong COM port | Check Windows Device Manager > Ports to find correct COM port number. |
| "Access denied" on COM port | Close all other programs using the COM port (Arduino IDE, PuTTY, Flash Tool). |

## 7.3 Board Not Responding After Flash

| Check | Action |
| --- | --- |
| GPIO0 still grounded? | Disconnect BOOT pin - must float high |
| Battery charged? | Measure with multimeter: 3.3-4.2V |
| Dual power conflict? | Use only ONE power source at a time |
| Firmware crash? | Open serial monitor to check for error messages during boot |
| Flash actually written? | Use chipInfoDump > Read Flash to verify data at 0x1000 (should start with 0xE9) |

# 8. Chip Reference

## 8.1 eFuse Summary

Your chip has the following eFuse configuration (all security features are disabled):

| eFuse | Status |
| --- | --- |
| Flash Encryption | Disabled (FLASH_CRYPT_CNT = 0) |
| Secure Boot V1 | Disabled (ABS_DONE_0 = False) |
| Secure Boot V2 | Disabled (ABS_DONE_1 = False) |
| UART Download | Enabled (UART_DOWNLOAD_DIS = False) |
| JTAG | Enabled (JTAG_DISABLE = False) |
| Bluetooth | Enabled (DISABLE_BT = False) |
| SPI Pad Config | CLK:6, Q:17, D:8, HD:11, CS:16 (PICO) |

## 8.2 Partition Table

| Name | Type | Offset | Size |
| --- | --- | --- | --- |
| nvs | data (nvs) | 0x009000 | 24 KB |
| phy_init | data (phy) | 0x00F000 | 4 KB |
| factory | app (factory) | 0x010000 | 1024 KB |

## 8.3 Key GPIO Assignments

| GPIO | Function | ESP32 Pin | Notes |
| --- | --- | --- | --- |
| GPIO26 | PWM2 (LCD Lens) | Pin 15 | Right lens dimming |
| GPIO27 | PWM1 (LCD Lens) | Pin 16 | Left lens dimming |
| GPIO4 | Hall Sensor Input | Pin 24 | Arm open/close detect |
| GPIO0 | Boot Mode Select | Pin 23 | LOW = download mode |
| U0_TXD | UART0 TX (Serial) | PAD9_2 | Serial output |
| U0_RXD | UART0 RX (Serial) | Pin 40 | Serial input |